Annotated Code Document

| Python Code | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns | This is importing python libraries that we will use to do the math in this project. |
| sns.set_style('whitegrid') sns.set_palette('deep') | This sets the background for all plots and also set a specific color palette. |
| nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') | This reads the raw data sets and then sets them to the specific name. |
| nhts_data = nhts_data.dropna(subset=['vehicle_type']) | This will remove data without vehicle data |
| data_grouped = nhts_data.groupby(['vehicle_type', 'census_region']).size().unstack(fill_value=0) | This will group the data sets of vehicle type and census region and then for any value not filled it will insert a 0. |
| plt.figure(figsize=(20,10)) data_grouped.plot(kind='bar', stacked=True) | This will first set the dimensions of the graph and then create a stacked bar chart. |
| plt.xlabel('Vehicle Type', fontsize=13) plt.ylabel('Count', fontsize=13) plt.title('Stacked Bar Chart of Vehicle Type by Census Region', fontsize=15) plt.xticks(rotation=90) plt.legend(title='Census Region', bbox_to_anchor=(1,1), loc='upper left') plt.show() | This code labels the x and y axis and creates a title. This also rotates the graph 90 degrees and creates a legend and also a location for the legend. The last time is what tells our code to render the plot that has been created. |
| plt.figure(figsize=(10,5)) | This will set the figure size for the plot. |
| sns.histplot(nhts_data['vehicle_age'], bins=30, kde=True, color='steelblue') | This creates a histogram chart that has 30 bins and is separated by the vehicle age. Then will make the bins steelblue. |
| plt.xlabel('Vehicle Age (years)', fontsize=12) plt.ylabel('Frequency', fontsize=12) plt.title('Distribution of Vehicle Age', fontsize=14) plt.show() | This code labels the x and y axis and creates a title. The last line is what tells our code to render the plot that has been created. |
| plt.figure(figsize=(12,6)) sns.boxplot(data=nhts_data, x='household_location', y='vehicles_per_household') | This code first sets the figure size. Then this code will create a box plot and label the x and y axsis based on their respective values. |
| plt.xlabel('Household Location', fontsize=12) plt.ylabel('Vehicles per Household', fontsize=12) plt.title('Distribution of Vehicles per Household by Household Location', fontsize=14) plt.xticks(rotation=45) plt.show() | This code labels the axis and the title as well as rotates the boxplot 45 degrees and finally calls to render the graph. |
| trajectory_number = 1 | This sets the trajectory number to one as a start to analyze. |
| data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number] plt.figure(figsize=(12,5)) | This filters the dataset to use only the slected trajectory. The last line will create the figure size. |
| sns.lineplot(x=data_subset['Time'], y=data_subset['leader_position(m)'], label='Leader Trajectory') | This is the graph of the leading car with position over time. |
| sns.lineplot(x=data_subset['Time'], y=data_subset['follower_position(m)'], label='Follower Trajectory') | This is the graph of the following car with position over time. |
| plt.xlabel('Time (seconds)', fontsize=12) plt.ylabel('Position (m)', fontsize=12) plt.title('Car Following: Trajectory Comparison', fontsize=14) plt.legend(fontsize=11) plt.show() | This code will label the graph and also create a title. And add a legend for readability. The function will then call to render the chart. |
| trajectory_number = 1 | This sets the trajectory number to one as a start to analyze. |
| data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number] plt.figure(figsize=(12,5)) | This filters the dataset to use only the slected trajectory. The last line will create the figure size. |
| scatter1 = plt.scatter( data_subset['Time'], data_subset['follower_position(m)'], c=data_subset['follower_speed(m/s)'], cmap='viridis', s=40, label='Follower' ) | This code will create a plot for the follower car as a position over time. This will call a variable c which is the color of the chart that indicates the speed of the vehicle. The cmap is the scheme for the color at which speeds are what colors. |
| scatter2 = plt.scatter( data_subset['Time'], data_subset['leader_position(m)'], c=data_subset['leader_speed(m/s)'], cmap='plasma', s=40, marker='s', label='Leader' ) | This code will create a plot for the leading car as a position over time. This will call a variable c which is the color of the chart that indicates the speed of the vehicle. The cmap is the scheme for the color at which speeds are what colors which is different for the leading car. |
| cbar1 = plt.colorbar(scatter1) cbar1.set_label('Follower Speed (m/s)', fontsize=12) | This adds and then labels the color bar that is being used to represent the follower car speed. |
| plt.xlabel('Time (seconds)', fontsize=12) plt.ylabel('Position (m)', fontsize=12) plt.title(f'Heatmap Trajectory Visualization for Trajectory {trajectory_number}', fontsize=14) plt.legend() plt.show() | This code will create labels for the graph and also create a title. Then we create a legend so that the reader can visualize what vehicle does what. |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) acceleration = a * (1 - (v / v0)**delta - (s_star / s)**2) return acceleration | This is the coded version of the IDM acceleration equation. The s_star equation is to calculate a safe following distance. The second s_star equation will ensure that are values will never be smaller than the safe following distance. The acceleration equation will solve for at which the vehicle is accelerating. This will be a function that we can call later to do calculations. |
| ngsim_data = pd.read_csv("NGSIM.csv") | Read and loads the dataset. |
| traj = ngsim_data[ngsim_data["trajectory_number"] == 1].copy() | Select a specific trajectory for the simulation. |
| leader_position = traj["leader_position(m)"].values leader_speed = traj["leader_speed(m/s)"].values follower_position = traj["follower_position(m)"].values follower_speed = traj["follower_speed(m/s)"].values | This code will pull the leader and follower data. |
| time = traj["Time"].values dt = time[1] - time[0] | The first line of code will pull all of the known time values and the second line will create a time step which is at the rate the time is changing. |
| v0 = 30 s0 = 2 T = 1.5 a_param = 1.0 b = 1.5 delta = 4 | These are values that we predetermine to run the simulation. |
| n_steps = len(time) | This will initialize the arrays. |
| sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acc = np.zeros(n_steps) | These arrays will store the simulated results. |
| sim_position[0] = follower_position[0] sim_speed[0] = follower_speed[0] | These are the initial conditions to our simulation. |
| for i in range(n_steps - 1): | Creates a loop to simulate motion. |
| s = leader_position[i] - sim_position[i] | Computes spacing of the vehicles at specific time values. |
| delta_v = sim_speed[i] - leader_speed[i] | This will determine if the following car is getting closer or further behind. |
| sim_acc[i] = idm_acceleration(sim_speed[i], s, delta_v, v0, s0, T, a_param, b, delta) | Computes acceleration. |
| sim_speed[i + 1] = sim_speed[i] + sim_acc[i] * dt | Updates speed using integration. |
| sim_position[i + 1] = sim_position[i] + sim_speed[i] * dt | Updates the position using integration. |
| print("Simulation complete!") | Pint showing the test is done. |
| plt.figure(figsize=(10,6)) plt.plot(time, leader_position, label="Leader Position") plt.plot(time, follower_position, label="Real Follower Position") plt.plot(time, sim_position, label="Simulated Follower Position", linestyle="--") plt.xlabel("Time (s)") plt.ylabel("Position (m)") plt.title("Vehicle Trajectory Simulation using IDM") plt.legend() plt.grid(True) plt.show() | This code will create the plot for the ibm simulation. This code will also create a legend and will label and place a title on the graph. |
| leader_acc = traj["leader_acc(m/s^2)"].values real_acc = traj["follower_acc(m/s^2)"].values | This is pulling the real data for the follower and the leader vehicles. |
| plt.figure(figsize=(10,6)) plt.plot(time, leader_acc, label="Leader Acceleration") plt.plot(time, real_acc, label="Real Follower Acceleration") plt.plot(time, sim_acc, label="Simulated Follower Acceleration", linestyle="--") | This uses the real acceleration of the leader and the follower and then create a third variable that is the simulated follower to see how close the real result is to the simulated. The codes also add labels to determine which model is which. |
| plt.xlabel("Time (s)") plt.ylabel("Acceleration (m/s^2)") plt.title("Acceleration vs Time") plt.legend() plt.grid(True) plt.show() | This code will create graph labels. And then will add a legend that will be able to read the values. |
| print(time_log) print() print("Total Minutes:", time_log['Minutes Spent (minutes)'].sum()) | This will print the individualized time log with time for each specific task and then add them all together and print a total time spent. |
| time_log = pd.DataFrame({ 'Task': [ 'Bar chart', 'Histogram', 'Boxplot', 'Time-series plot 1', 'Time-series plot 2', 'Customized plot', 'IDM simulation setup', 'IDM trajectory plot', 'IDM acceleration comparison plot', 'Write-up / interpretation' ], | This code creates a time log by adding up all the time spend on each specific task. |